Alan Li

EE 371

February 12th, 2022

Lab 3 Report

# Task1

## Section 1: Procedure

My approach to draw a line for task 1 is to display all pixels on the line one by one at each clock edge. There are 3 modules for task1, DE1_SoC is top-level module, VGA_framebuffer has already been designed. I implement a line_drawer module to finish the task. The algorithms for developing line_drawer can be found here: http://members.chello.at/easyfilter/bresenham.html. I did some modifications since the algorithm is originally written in C which is a programming language. There is some differences between programing languages and VHDL. I put the lines inside for loop of c code in a always_ff block, in that way the registers will get updated at each rising clock edge. I put everything else inside an always_comb block.

## Section 2: Results

Line_drawer testbench

For line_drawer testbench, I set x0, y0, x1, y1 as 100,100,150,300. After reset, both x and y are increasing at different rates because the slope is not 45 degrees. After hundreds of clock cycles, xPointer and yPointer has reached the destination coordinates and will stop there like displayed below.

## Section 3: Appendix

# Task2

## Section 1: Procedure

I’m using the same lilne_drawer module as task1, for task2 I added a line_animation module that make the line move around the screen. The module periodically updates the start and end point. I implemented a module of a three-state FSM. The state diagram is given below.

After the counter reaches the maximum, the drawing would be finished, the system goes to the state of refresh and redraws the same path but with color black. Before moving to the next drawing state, the system updates the new coordinate and makes sure that the coordinate stays within the bound. If the reset switch is enabled, the module goes immediately from drawing to refreshing. After clearing the screen, the system stays at reset until the reset switch is off.

## Section 2: Results

From the simulation for line animation, you can see that the x0, y0 is getting updated. At the same time, the color is toggling between 1 and 0.

The simulation for line_drawer is the same as task1

## Section 3: Appendix